LAPORAN JARINGAN SENSOR NIRKABEL

PROTOTYPE LAMPU CERDAS

OLEH:

Muh. Khaeril Syam		(D42116008)

Lutfi Qadri			 	(D42116012)

Andi Muh. Agung Alif H.	(D42116502)

Dandi Wisnu Shoreandi		(D42116512)

DEPARTEMEN TEKNIK INFORMATIKA

FAKULTAS TEKNIK

UNIVERSITAS HASANUDDIN

2019

- Alat dan Bahan

- NodeMCU V3 (2)

- Sensor Ultrasonic

- Relay Module 1 channel

- Kabel / Jumper

- Kabel USB (2)

- Lampu

- Aplikasi Blynk

- Langkah-langkah Pengerjaan

- Rangkaikan NodeMCU, Relay dan lampu menggunakan kabel/jumper.

NodeMCU			Relay				Lampu

Pin 3V				VCC				-

Pin D4				Pin IN				-

Pin GND			GND				-

-				NO				Anoda Lampu

-				COM				Sumber tegangan

- Rangkaian NodeMCU dan sensor ultrasonic menggunakan kabel/jumper

NodeMCU			Sensor Ultrasonic

Pin D3				Pin Echo

Pin D4				Pin Trig

Pin 3V				Pin VCC

Pin GND			Pin GND

- Selanjutnya Upload masing-masing sketch program ke NodeMCU

- Selanjutnya rangkaiakan masing-masing alat dan bahan yang sesuai dengan masing-masing letak fungsinya.

- Atur Pin, Acces Point dan token pada aplikasi Blynk.

- Cara kerja alat

Alat ini bekerja sebagai otomatisasi lampu, dengan menggunakan sensor ultrasonic sebagai sensor utama dan menggunakan dua boar mikrokontroller jenis NodeMCU, dimana salah satu NodeMCU terhubung dengan sensor dan NodeMCU yang lain terhubung dengan relay yang akan mengaktifkan lampu, jadi jika sensor ultrasonic mendeteksi pergerakan maka data akan terkirim ke NodeMCU dan mengaktifkan lampu.

- Skema rangkaian

- NodeMCU, relay dan lampu

- NodeMCU dan sensor ultrasonic

- Hasil

Lampu mati

Lampu menyala

NodeMCU, relay dan lampu

- NodeMCU, sensor ultrasonic

- Flowchart

- Tampilan pada aplikasi Blynk

- Token di Surat elektronik

- Token NodeMCU, sensor ultrasonic (sebagai server)

- Token NodeMCU, Relay dan Lampu (sebagai client)

- Source Program

#define BLYNK_PRINT Serial

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

// You should get Auth Token in the Blynk App.

// Go to the Project Settings (nut icon).

char auth[] = "363a0940b4564ef0b123629937006257";

// Your WiFi credentials.

// Set password to "" for open networks.

char ssid[] = "acer";

char pass[] = "sayasaya";

bool touched = false;

void setup()

{

// Debug console

Serial.begin(9600);

pinMode(2,OUTPUT);

digitalWrite(2,00);

Blynk.begin(auth, ssid, pass);

}

void loop()

{

Blynk.run();

}

BLYNK_WRITE(V1)

{

int pinValue = param.asInt(); // assigning incoming value from pin V1 to a variable

AturRelay(pinValue);

// process received value

}

void AturRelay(int pinValue){

if(pinValue == 1){

digitalWrite(2,1);

Blynk.notify("LAMPU MENYALA");

//     digitalWrite(2,HIGH);

}//NYALA

else if(pinValue == 0){

digitalWrite(2,00);

Blynk.notify("LAMPU MATI");

//     digitalWrite(2,LOW);

}//MATI

}

NodeMCU, relay dan lampu (sebagai client)

#define BLYNK_PRINT Serial

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

// You should get Auth Token in the Blynk App.

// Go to the Project Settings (nut icon).

//DEFINE PINS

const int trigPin = 2; //D4

const int echoPin = 0; //D3

//DEFINE VARIABLES

long duration, distance;

char auth[] = "8332411289ea46ccae43f96799072c0f";

// Your WiFi credentials.

// Set password to "" for open networks.

char ssid[] = "acer";

char pass[] = "sayasaya";

bool switches = false; // false = relay mati (HIGH) , true = relay nyala (LOW)

WidgetBridge bridge1(V1);

// Timer for blynking

BlynkTimer timer; BLYNK_READ(V1)

{

// This command writes Arduino's uptime in seconds to Virtual Pin (5)

}

void setup()

{

pinMode(trigPin, OUTPUT); // Sets the trigPin as an Output

pinMode(echoPin, INPUT); // Sets the echoPin as an Input

// Debug console

Serial.begin(9600);

Blynk.begin(auth, ssid, pass); timer.setInterval(1000L, blynkAnotherDevice);

}

void loop()

{

Blynk.run(); timer.run();

}

void blynkAnotherDevice() // Here we will send HIGH or LOW once per second

{

// Clears the trigPin

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

// Sets the trigPin on HIGH state for 10 micro seconds

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

// Calculating the distance

distance = duration*0.034/2;

Serial.println(distance);

if(distance > 50){ // jika nilai lebih dari 50 maka lampu mati

//      Serial.println(SensorValue);

bridge1.digitalWrite(V1, HIGH);  //

bridge1.virtualWrite(V1,1);

}else if(distance < 51 ){ // jika nilai lebih dari 50 maka lampu menyala

bridge1.digitalWrite(V1, LOW);

bridge1.virtualWrite(V1,0);

}

}

BLYNK_CONNECTED() {

bridge1.setAuthToken("363a0940b4564ef0b123629937006257");

// Place the AuthToken of the second hardware here

}

NodeMCU, Sensor Ultrasonic (sebagai server)